- Scope

This procedure applies to all forest land not currently planted to commercial species, and includes:

- Natural habitats

- Special Conservation Sites

- Maintained areas

- Riparian zones and Wetlands

- Wildlife

- Cultural, Historical, Archaeological, Paleontological sites

- Ecological processes,

- Escapee weeds on adjacent land

- Erosion Control

- Objective

The objective is to optimise land dedicated to Special Management Zones (SMZ’s) through effective planning in such a way that operations are done safely and do not cause land degradation, affect the productivity of management, and mitigate the impacts of these operations on the aspects of the environment.

- Legal Compliance Requirements

Field operations shall comply with all national laws and codes of practice.  Where no such code exists, the operation shall comply with the “ILO - Code of Practice on Safety and Health in Forestry” and IFC Standards

- Operational Control

| 1 | Every crew, of more than five people, working in field shall have a first aider present in Field. | S |
| --- | --- | --- |
| 2 | First Aiders shall have a first aid kit with them in-field that complies with the company requirements. | S |
| 3 | Employees shall be represented by a SHE representative. | S |
| 4 | Team briefings / team talks / toolbox talks shall be done. | Q |
| 5 | Daily Production and labour details shall be recorded on the PCS form. | P |
| 6 | Where the Group SHEQ Manager does not have the proficiency to deal with any particular aspect, use shall be made of expert assistance, be it internal or external. Such proficiency should be measured in terms of academic qualifications, training and experience. | Q |
| 7 | Decisions made in terms of these prescriptions shall be included in the Plantation Management Plan. | Q |
| 8 | Sufficient supplies of safe drinking water should be available at the worksite. For physical work in hot climates up to 1 litre per hour per worker may be required. | S |
| 9 | Non-organic waste shall be disposed of in the correct manner. (Bins/bags shall be provided for such waste disposal). | E |
| 10 | Sun protection (hats) are to be provided to workers in field to protect against UV radiation in hot climates. | S |
| 11 | Smoking shall be restricted to designated areas, as defined by the company. | S E |
| 12 | In the event of a hydrocarbon (fuels and oils) spill refer to the oils spills BOP. | E |
| 13 | Refer to the Preventative Action, Corrective Action procedures, and CAR Form where non-compliances to this prescription are observed. | Q |

  - General Prescriptions

| 1 | Where necessary outside expertise may be used to assist in identification and management prescriptions of conservation areas. | Q |
| --- | --- | --- |
| 2 | Where possible roads should be established to demarcate the boundary between an SMZ and the commercially planted area. Such new roads shall comply with the requirements of Road planning and Construction FMP’s, Environmental Impact Assessment FMP and an Environmental Management Plan shall be done. | Q |
| 3 | Planning shall be done on an integrated basis: - Major SMZ’s shall be linked through corridors to provide for some movement of organisms from one ecological zone to another. - buffers strips should be as wide as possible, and planned on a large landscape level to ensure continuity and flow. - conservation areas should not occur in isolation. | E |
| 4 | Any road requirements in terms of harvesting operations should be determined before commercial planting takes place and if a road is to be established on the edge of an SMZ, it should be constructed before such planting takes place. Such roads should not be placed within the buffer zone as determined for the particular site, unless specified otherwise. Where possible, roads that are detrimentally affecting the functioning of SMZ should be removed or diverted to improve ecological functioning of the SMZ. | Q |
| 5 | Planning for burning shall be done to provide mosaics of vegetation in terms of age, structure and composition. | E |
| 6 | No standard can be given for the effective planning, but monitoring of the implementation thereof should highlight success and failures that can be used when planning is reviewed. | Q |
| 7 | Open Area management shall be scheduled in the 5 year open area management plan. Such a plan shall include noxious weed control and this should be: - Integrated in terms of different methods applied (Non-chemical methods should be considered, where practicable); - Systematic, in terms of initial control and follow-up; - Long-term, in order to set goals and measure progress; - Holistic, in terms of incorporating the entire plantation (open areas and compartments); - Scheduled annually according to priority from the weed control management plan and budget. (Refer to the Integrated Weed Management Guideline for more details) | Q |
| 8 | When scheduling noxious weed control operations consider: High Priority – start with: areas with low alien / invasive plant numbers, Commercial areas scheduled for normal forest operations, Clearing on a catchment basis, the top end of water courses and the top of slopes, i.e. work down water courses and down slopes or catchments. Complete follow-up of control work done the previous year. Newly disturbed areas e.g. cultivation and uncontrolled wild fires. Edges of dense spreading infestations, confine the area by clearing 10 metres wide belts around them (See diagram below). Low density infestations inside dense infestations (See diagram below). Also consider the land use, and pioritise areas, that are of high value in terms of high conservation value, water catchment use, cultural / historical sites where stakeholders have requested clearing, and high aesthetic value e.g. along public roads, and around tourist sites. Medium priority - similar to high priority, but at lesser levels, or of lower land use value. Low priority – Stabilised areas, Areas where dense infestations could not become worse, Areas where alien plants have little or no impact e.g. on water yield, or will not spread to other areas. For example, Rock outcrops and open areas in the middle of compartments. These areas should be scheduled for weeding during normal forest operations in the compartments, and shall therefore not form part of any weed management 5 year and APO’s. | QE |
| 9 | Refer to the integrated pest management plan for identified pests and their treatment. | Q |
| 10 | Each plantation shall identify compartment (both commercial and open) infestation levels, both intensity and weed species), initially this can be subjective based on forest management observed infestation levels. Infestation levels shall be based on the following criteria: - Maintenance = weed infestation levels <5 % - Low infestation = weed infestation levels 6% - 25% - Medium infestation = weed infestation levels 26 – 50% High infestation = weed infestation levels >50% | QE |
| 11 | Following the completion of weeding operations a post weed control assessment shall be done 6 weeks after control. | Q |
| 12 | Annually update weed infestation level data (Weed infestation registers) & maps based on post control assessments, and subjective assessment of areas not done in the year. | Q |
| 13 | Revise existing commercial and open area management plans on an annual basis to take into account the infestation levels and priority levels where it is determined necessary. The critical element in the plans should be to follow up on previous weeding and ensure eradication of noxious weeds, and to clear areas / blocks / catchments. | EQ |
| 14 | In annual operational / plantation / country reports, report on actual verses achieved, justify changes made to the plans. | Q |
| 15 | In the annual sustainability report on noxious weed infestation level change and achievement of actual to planned reductions. | Q |
| 16 | Refer to the Non-Timber Forest Products for prescriptions on utilization on resources for natural areas. | EQ |
| 17 | Where an area of a new plantation establishment has been degraded by previous land use (before the company acquired the land) and this area is either within a legislated buffer (for example a riparian or lake shore buffer) this area should be protected in a natural state to facilitate natural recovery. | E |
| 18 | The area should have all invasive exotic species controlled through methods identified under points 8 through to 13 above. Commercial crops (for example banana, cassava, maize, jack fruit should either be burnt or slashed to ensure effective elimination of these agricultural crops in these natural areas. | E |
| 19 | Effectiveness of control and rehabilitation of these areas shall be assessed as part of the biodiversity studies conducted every five to ten years. | E |
| 20 | Where forest plantation crops have been accidently established within these buffers these buffers shall be demarcated on plantation maps and on the ground following GPS work. | E |
| 21 | At the time of the next harvesting operation, either thinning or clearfelling these demarcated areas shall be cleared of all plantation species, and the area shall be maintained as part of the open area management plan until such time as it achieves a maintenance phase status (< 5 % infestation with exotic species) | E |
| 22 | Effectiveness of control and rehabilitation of these areas shall be assessed as part of the biodiversity studies conducted every five to ten years. | E |

  - Conservation Management Planning

| 1 | The conservation management shall for part of the Forest Management Plan. This plan shall contain at least the following information: - Description of the physical, biotic and a list of interested and affected party. - Environmental Management data required such as the Land Cover Class, Land Management Units, Areas of Special Interest and species data lists. - Management prescriptions and schedules such as burning and weed control programmes (refer Tending FMP for weed control planning), excision schedules, waste management and fauna and flora management. - Monitoring requirements. | Q |
| --- | --- | --- |
| 2 | The Plantation Manager shall monitor on at least an annual basis that the plan is executed. | Q |
| 3 | The General Manager shall ensure that the requirements of the plan are included in the APO and monitor progress on a monthly basis. | Q |
| 4 | The Plantation Manager shall include the relevant planning actions in the APO and ensure that these are executed. | Q |
| 5 | The Plantation Manager shall ensure that any excisions of the planted area implemented in terms of the plan are reported to the Planning Manager with annual updates. | Q |

  - Biodiversity Prescriptions

| 1. | Biodiversity studies shall be conducted on all plantations and reviewed against existing baselines on a 5 to 10-year basis. | E |
| --- | --- | --- |
| 2. | The Group SHEQ Manager shall co-ordinate biodiversity studies in the Group. | E |
| 3. | The Forestry Managers shall be responsible for ensuring that initial biodiversity studies and reviews of current baselines are completed based on point 1 above. | E |
| 4. | Biodiversity studies should be conducted by local specialists, such as at National Universities, normally conducted under the leadership of one specialist. | E |
| 5. | See Terms of Reference for initial and follow up biodiversity that can be used as a guide with lead consultants conducting these studies. | E |
| 6. | Biodiversity reports shall be kept in hard copy on the plantation, with soft copies in the Natural Resources Management Library section in the IMS. | E |
| 7. | Monitoring of HCV Permanent Sample Plots (PSP’s) shall be scheduled and done as part of the Biodiversity reviews every 5 to 10 years. | E |

  - Indigenous Forest and Protected Species Prescriptions

| 1 | The identification of areas of intact woodland and degraded ex woodland should occur during planning phase. By the removal of the large, primary and valuable tree component from the intact woodlands, the functioning of the remaining degraded woodlands has changed. When degraded, what remains is a grassland/shrub dominated vegetation. The Company regards intact woodland as forest and therefore protected (no afforestation), while degraded ex woodland can be planted. Refer to Table 1. Below for guidance to decide if an area is degraded or not. | E |
| --- | --- | --- |
|  | Table 1. Criteria for identification of afforestation areas. Criteria 1; Criteria 2; Criteria 3; Afforestation/Protection potential. Apply criteria from left to right. Vegetation type which includes trees as part of the habitat. (A “tree” is defined as “A perennial plant having a self-supporting woody main stem or trunk (which usually develops woody branches at some distance from the ground), and growing to a considerable height and size.); At least two indigenous trees whose crowns are largely contiguous).; Forest in good condition with understory layer.; Intact Indigenous Forest. Protect from afforestation and manage as conservation area. Develop appropriate buffers.. Forest with poor understory and signs of past damage.; Degraded Indigenous Forest. Protect from afforestation and manage as conservation area. Develop appropriate buffers.. Single trees in a bushveld/woodland/grassland landscape).; Woodland dominated by trees.; Intact woodland. Protect from afforestation and manage as conservation area. Develop appropriate buffers.. Woodland, not dominated by trees, but consisting notably of a shrub component (and often showing signs of disturbance).; Degraded ex woodland. Consider for afforestation, but also leave some representative portions and corridors on the plantation as part of holistic planning.. Grassland with few small trees and shrubs.; Grassland. Consider for afforestation, but also leave some representative portions and corridors on the plantation as part of holistic planning.. Note: As vegetation often changes over space and is not always an absolute with no obvious cutoff, it could in some areas be very difficult to rate the areas with absolute accuracy as degraded woodland or not, and therefore for these areas, meaningful cut offs need to be developed. Consider landscape and the consolidation of boundaries in these areas.. |  |
| 2 | No tree or forest protected in terms of legislation or FSC definition of Forest shall be removed or damaged. | E |
| 3 | Where doubt exists as to the status of a tree, group of trees or forest, the Plantation Manager shall ascertain this through consultation with the Group SHEQ Manager. At no time shall forest be removed as this will result in illegal conversion of forest | Q |
| 4 | All indigenous forests are regarded as conservation areas. No commercial exploitation of trees shall occur in these forests. | QE |
| 5 | Natural forest margins: Depending on the forest type, the type of adjoining vegetation, and whether disturbance processes from outside the forest are natural or not, natural forests adjoining other vegetation types have margins generally forming an ecotone or transitional zone between forest and woodland or grassland. - Any management intervention should be aimed at maintaining or simulating natural processes, e.g. periodic burning of fire prone vegetation like grassland. - Artificially protecting forest edges by making fire belts around them, or lighting fires from the forest edge, when controlled burns are made, are practices detrimental to the forest margin in the long run, and are therefore not allowed. - Frequency and season of controlled burns shall follow the current scientifically based prescriptions, which are accepted as most closely simulating natural processes for local environments. | E |
| 6 | Artificial margins: Artificial margins are those bounding on exotic plantations, roads and other artificial features: - Where forest margins bound on roads or plantations, and smaller pockets of plantation compartments occurs between the road and the forest edge, causing the forest edge to have a wavy or finger-like margin, this should be eliminated by establishing/re-establishing forest in these in order to straighten the margin to align it with the road. - Road drains need to be sited in such a way that water run-off from roads drains into natural drainage channels; otherwise the changed micro-habitat may cause a decline in the natural forest cover in patches. - No slash, rubble, or waste road material may be dumped in the natural forest beyond the roadside. | E |
| 7 | Weed control: As prescribed under section 4.1 steps 8-15). | QE |
| 8 | A buffer strip of a minimum of 5m, shall be left unplanted during initial planting or after clear-felling of a mature plantation stand bordering on an indigenous forest. | PEQ |
| 9 | For areas identified that do not comply with the buffer zone prescription, a CAR shall be raised with the aim of addressing such at the next practical opportunity - such as during harvesting of the mature compartment. | E |
| 10 | In cases where the determined buffer zone reaches close to an existing road, the road shall be used as the end of the buffer zone where practically possible as determined by the plantation forester. | E |

  - Riparian Zones and Wetlands Prescriptions

| 1 | Riparian zones and wetlands shall be identified and delineated using country legislation as a minimum requirement, however where country legislation does not exists or does not speak clearly about the issue, NFC will adopt the following prescriptions: - Perennial rivers: Rivers that have a constant stream-flow throughout the year: - Uganda: 50m (on both sides) - Tanzania: 60m from the river bank (on both sides) - Rwanda: 10m (on both sides) - Periodic rivers: Rivers that run dry on occasion: - Uganda: 20m (on both sides) - Tanzania: 20m (on both sides) - Rwanda: 5m (on both sides) - Episodic/Intermittent rivers: Rarely occurring rivers formed from runoff channels: - Uganda: 5m (on both sides) - Tanzania: 5m (on both sides) - Rwanda: 5m (on both sides) - Wetlands: Land consisting of marshes or swamps: - These areas should be delineated according to the presence of mottling - Uganda: 10m (on both sides) - Tanzania: 10m (on both sides) - Rwanda: 10m (on both sides) | E |
| --- | --- | --- |
| 2 | This will initially be done as a GIS mapping exercise that is then verified by the forester infield in compliance with the Land Use Classification MP. | Q |
| 3 | On completion of delineation the forester shall review the delineation data and map. This data shall be verified and where he/she is satisfied with the new boundaries they shall sign off the data and forward to planning for GIS capture and finalisation. | Q |
| 4 | Where there are queries in the delineation boundaries these shall be noted for verification with the Group SHEQ Manager or Planning Manager. | Q |
| 5 | During internal audits, the delineation process shall be audited to ensure compliance to these prescriptions. | Q |
| 6 | Any deviation from the guidelines shall be corrected as per the audit report recommendations. | Q |
| 7 | Weed control: As prescribed under section 4.1 steps 8-15, Management other than weed control shall consider the ultimate vegetation cover that is desired within the SMZ. | QE |
| 8 | Where a grass or herbaceous cover is required, a regular burning programme shall be prescribed. | QE |
| 9 | Any drainage from a manmade structure shall drain through sufficient buffer of vegetation (10 meters) or other means of filtering before entering a water course | E |
| 10 | Burning may be allowed in wetlands, but only on a scientifically recognised rotation. This shall be scientifically determined before any burning can occur. | E |
| 11 | As wetlands and rivers are regarded as an important source of water to the catchment and local area, no multiple resource use should be considered in wetlands where cultivation is considered. Resource use such as rice farming and cultivation of crops, affect water quality, and effects downstream users. Where this is occurring, it shall be addressed. The use of wetlands and rivers to provide reeds, sedges, grasses and medicinal plants is however seen as acceptable, and could occur, provided it is permitted and controlled through enforcement on the plantation. | E |
| 12 | Where appropriate, research shall be carried out as recommended for identified stream and water course indicators to determine baselines for future monitoring. | E |

  - Grasslands, woodlands and other natural vegetation Prescriptions

| 1 | Burning regimes for natural vegetation shall where necessary be determined by one or more of the following; - Outside expert assistance. - Legal requirements in terms of season of burn - Climax vegetation that is to be established or maintained. - Breeding cycles and season of fauna that inhabits the vegetation. - Fire protection requirements of the plantation. - natural fires | QE |
| --- | --- | --- |
| 2 | The monitoring of the species composition, trends in vegetation change, species richness and fuel loads will assist management in determining what frequency of burning to implement and also when to burn as well as what type of burn is needed. A systematic programme to determine the acceptable burning rotation must be developed before burning rotations are finalised. | E |
| 3 | Weed control: As prescribed for Riparian Zones and Wetlands (Refer section 4.1 step 8-15). This may need to be synchronised with the fire management regimes | QE |
| 4 | Erosion in grassland and woodlands occurs due to disturbance (roads, overgrazing) and rehabilitation measures need to be introduced where this occurs. As far as possible only indigenous plants must be used for stabilization measures. | E |
| 5 | Grasslands and woodlands experience intense pressure from grazing by agricultural stock. A need to assist local communities is however recognised. All grazing on plantations shall be controlled and the permitting procedures introduced and enforced on plantations | E |
| 6 | All multiple resource harvesting shall take place through a permit system with active enforcement. Resource harvesting should only occur as part of subsistence assistance to local communities and no commercial resource harvesting is allowed. | E |

  - Areas of Special Interest (ASI’s) Prescriptions

| 1 | The following elements shall be classified as ASI’s. - Paleontological objects and sites - Archaeological sites - Historical sites, structures, buildings and ruins - Cultural or historical landscapes, such as sacred forests or big trees. - Caves - Waterfalls - Scenic view points - Vegetation that has historical value or special or cultural interest. - Tree with active raptor nests - Burial sites and cemeteries / Graves - Dams or other water bodies not included as a riparian SMZ. - Declared National Monuments | Q |
| --- | --- | --- |
| 2 | ASI’s thus identified shall be provided with the following buffers. - A buffer of at least 5 metres shall be maintained around ASIs. A road may be included as part of this buffer. - If significant damage is inevitable during operational activities, this distance should be increased to mitigate such impacts. | QE |
| 3 | Planting next to existing public view sites or significant aesthetic sites shall not impede such views throughout the rotation of the particular commercial stand(s). | QE |
| 4 | Where any operational activities may impact on such sites, a specific buffer shall be established and recorded within the Areas of Special Interest (ASI) record of the site and this zone shall need to be sufficient to exclude such potential impacts. | QE |
| 5 | No ASI shall be destroyed or altered in any way, without the approval of the Forestry Manager. | QE |
| 6 | A record of every ASI shall be included in the CMP and the ASI Record used as a pro-forma. | Q |
| 7 | Where necessary outside expertise may be used to assist in determining management prescriptions. | Q |
| 8 | The management prescriptions shall be defined in the ASI record. | Q |
| 9 | Prescriptions may include: - No intervention, where conditions are relatively stable and no immediate threats are perceived. - Preservation, including short-term protective measures as well as long-term actions to retard deterioration or prevent further damage. - Repair and Restoration. - Eradication of alien plants on the site. | Q |
| 10 | Communities and the work force shall be informed about the existence of Significant Sites, and the importance of reporting any new findings. The management of Significant Sites shall have the approval of local communities where applicable. | Q |

  - Fauna and Flora Prescriptions

| 1 | Independent specialist surveys must be undertaken to investigate the presence of rare, threatened and endangered fauna and flora occurring on the plantation. | QE |
| --- | --- | --- |
| 2 | A list of fauna and flora occurring on the forest shall be maintained by the Plantation Manager. | Q |
| 3 | All personnel should be encouraged to participate in identifying and monitoring these species. Where feasible a pictorial guide should be developed for the identification of all rare, threatened and endangered species, and staff trained in the identification of these species. | Q |
| 4 | The Species Record shall be used as a pro-forma. | Q |
| 5 | This species record shall be up dated as information becomes available. | Q |
| 6 | Rare threatened and endangered species shall be identified in the species record. | Q |
| 7 | The CITES (www.cites.org) and IUCN Red Data lists (www.iucnredlist.org) may be used as reference documents to assist in determining Red Data species. | Q |
| 8 | Management prescriptions for SMZ’s as discussed elsewhere in this procedure, shall take into account the requirements of fauna and flora. This shall include a programme to monitor and manage the rare, threatened and endangered species in the PMP. Priority shall be given to high risk species: - Species with limited distributions - Species with small home-ranges, or which do not disperse large distances - Species with specialised habitat requirements Species directly threatened by forestry activities or multiple resource utilisation in the past | Q |
| 9 | Protected species shall not be utilised in any form without the required permit from the relevant authority. | E |
| 10 | Species not protected shall not be utilised in any form without the written approval of the Forestry Manager. | E |
| 11 | The Forestry Manager shall only approve utilisation mentioned in step 10 above if information is available on the population(s) of the specie(s) and the impact of such utilisation can be quantified. | E |
| 12 | Where monitoring indicates that management practices have a negative impact on species, the Forestry Manager shall introduce alternatives management prescriptions to: - mitigate such impacts on species and to, - stop any impacts on species with special protection status. The PMP shall be updated in terms of such alterations. | Q |
| 13 | Plantation and indigenous forest trees with active nest of birds of prey shall be left in a buffer of surrounding trees and recorded, mapped, marked infield and should not be disturbed. Plantation compartments with known nesting sites should not be harvested during the breeding season. | E |

  - Escapee Weed Control Prescriptions

| 1 | Control of escapee weeds shall be included as part of Forest Management Planning, including: - recording all commercial species on neighbouring property which originate from the plantation - specific provision for the eradication of the spread of the commercial tree, alien weed species, grown on that plantation - proactive control of commercial specie spread. - Identification of specific locations on neighbouring property of escapee trees and plans for the eradication of such trees. - timeous follow-ups to minimise the spread of escapee weeds. | Q |
| --- | --- | --- |
| 2 | Refer to sections 4.1. and 4.2 of the Tending FMP for weed management prescriptions. | Q |

  - Erosion control Prescriptions

| 1 | Erosion prevention measures are covered in many prescriptions from planning through to corrective action, monitoring and reporting. These prescriptions shall be followed where each of these activities are undertaken on company land. | E |
| --- | --- | --- |
| 2 | Where forester, managers and contractors see erosion occurring, this shall be reported to the Plantation Manager. | E |
| 3 | The Plantation Manager shall assess the erosion site and record the details on the Erosion Site Record, including identified remedial action necessary to prevent further erosion on the site. Photos shall be taken at the time of identification and during erosion site assessments, that shall be done at least once every six months until the site is closed | EQ |
| 4 | Monitoring of erosion control on identified sites shall be recorded in the Erosion Monitoring Register until such time as the site can be considered closed. | EQ |
| 5 | Plantation Managers shall report erosion control at the time of site identification; assessment, and; closure in regular plantation reports to the executive management. | Q |
| 6 | Erosion control shall form part of the company’s annual sustainability report. | Q |

  - Conservation Management Monitoring Prescriptions

| 1 | The Forestry Manager is directly responsible for all biological-monitoring programmes in the region. | Q |
| --- | --- | --- |
|  | A programme that details monitoring shall be budgeted for by the Plantation Manager, and included in the PMP and Open Area Management Plan. | Q |
| 2 | Such monitoring programmes will be formalised, provided in the PMP and Open Area Management Plans as per section 4.1 point 8-15. | Q |
| 3 | Results from the monitoring shall be fed back into decisions in terms of management prescriptions and these be adjusted, as results become available. | Q |
| 4 | Management prescriptions and planning shall be adjusted, where required, to provide for the results of such monitoring. | Q |
| 5 | The Forestry Manager shall ensure that this information is included the PMP. | Q |
| 6 | The Forester shall monitor weed infestation in SMZ’s and complete the Open Area Management Plan accordingly. | Q |
| 7 | The Forestry Manager shall on an annual basis, monitor that this is done. | Q |
| 8 | Any deviation from the prescriptions in the PMP shall be authorised by the Forestry Manager. | Q |
| 9 | The Plantation Manager is responsible for the implementation of the PMP. | Q |
| 10 | The Plantation Manager shall monitor development and implementation of the 5 year weed control plans and annual implementation and completion of this plan | Q |

- Responsibility

The following positions are responsible for ensuring compliance with this procedure:

| Overall: | PLANTATION MANAGER |
| --- | --- |
| Development of 5 year conservation plan: | PLANTATION MANAGER |
| Annual weed planning and implementation: | PLANTATION MANAGER |
| Identification, Monitoring & Recording of RTE species | APPOINTED SPECIALIST |
| Records and Monitoring programmes for RTE species | PLANTATION MANAGER |
| Maintenance of Sightings Register for plantation. | PLANTATION MANAGER |
| Implementation of monitoring programme | PLANTATION MANAGER |

- Appendix

- Records

| Record | Responsibility | Where kept | Retention |
| --- | --- | --- | --- |
| Field data and maps for Conservation Management Plan | Plantation Manager | Planning Office | 5 years |
| Conservation Management Plan | Plantation Manager | Conservation Files | 5 years |
| Environmental Monitoring Reports and CAR’s | Plantation Manager | Conservation Files | 2 years |
| Schedule of Conservation Management Planning | Plantation Manager | Conservation Files | 2 years |
| Digital Data Base | Planning Manager | Planning Computer | Indefinite |
| Licenses, permits | Plantation Manager | Conservation Files | Indefinite |
| Biological monitoring methods and procedures | Plantation Manager | Conservation Files | Indefinite |
| Species Monitoring | Plantation Manager | Plantation Office | Indefinite |
| ASI Register | Plantation Manager | Plantation Office | Indefinite |
| Plantation Working Plan | Plantation Manager | Conservation Files | 2 years |
| Information on status of protected species | Plantation Manager | Conservation Files | Indefinite |
| Plantation audits | Plantation Manager | Plantation | 5 years |
| Five year Weed Plan | Plantation Manager | Plantation Office | Indefinite |
| Annual Plan of Operations and record of work completed | Plantation Manager | Plantation Office | indefinite |
| Erosion Site Record | Plantation Manager | Conservation Files | 5 years |
| Erosion Monitoring Register | Plantation Manager | Conservation Files | 5 years |

- References

| IMS Manual | OHSAS 18001:2007 | ISO 14001:2004 | ISO 9001:2000 |
| --- | --- | --- | --- |
| 3.3.6.5 | 4.4.6 | 4.4.6 | 7.1; 7.2.1; 7.2.2; 7.3.1/23/45/6/7; 7.5.1/2 |
| FSC | NOSA 2008 | Aspect /Impact/Hazard Register | BOP |
| 3.3; 4.4; 6.1/2/3/6/9; 9.2/3; 10.1/7 |  | Impacts MC110 Risks |  |

Other:

CITES Lists

IUCN Red Lists – www.iucnredlist.org

Relevant in country legislation

- Definitions

  - Special Management Zone (SMZ): Areas within the plantation which contain features that are natural or manmade, and which require particular management strategies due to their scientific, ecological, palaeontological, archaeological, historical, traditional, aesthetic or recreational values.  Fire breaks and power lines are not special management zones, unless they have the characteristics that define them as SMZs

  - Natural Forest (FSC Definition):  Forest areas where many of the principal characteristics and key elements of native ecosystems such as complexity, structure and diversity are present, as defined by FSC approved national and regional standards of forest management.